9th International Congress of the Serbian Society of Mechanics

July 5-7, 2023, Vrnjačka Banja, Serbia

FORCED VIBRATIONS WITH BURGERS TYPE OF DAMPING

Dragan T. Spasić

University of Novi Sad, Faculty of Technical Sciences, Department of Mechanics,

Trg Dositeja Obradovića 6, 21000 Novi Sad, Serbia

e-mail: spasic@uns.ac.rs

Abstract:

This paper deals with energy dissipation during forced vibrations of a simple mechanical system. It combines the Newton axiom, the Newton-Laplace principle and a constitutive axiom corresponding to the classical Burgers type of viscoelastic body in which the internal friction occurs. Despite the fact that this constitutive equation relates force and its derivatives, on one hand side, with velocity and acceleration, on the other, the problem belongs to the classical Newtonian mechanics. As an example, the energy dissipation within the Burgers structural element under the action of the Ricker type disturbing force was estimated by use of the Laplace transform method.

Key words: Forced vibrations, Burgers element, Ricker wavelet

1. Introduction and preparatory results

Energy loss is a very important issue, no matter if it has to be minimized (to reduce cost) or maximized to reduce damages during impact or seismic activities, or designed to maintain some stability or control criteria. In mechanics it appears in various viscoelastic elements springs commonly used in conjunction with dashpots. Besides energy dissipation these elements have a role to limit the motion between the elements of the mechanical system. Some of the classical examples are known as Kelvin-Voigt, Kelvin-Zener, Burgers, anti-Zener model of viscoelastic body, see [1].

In what follows we shall consider the classical Burgers element, comprising 4 parameters: two elasticities  and two viscosities ,  see Fig. 1. Let us recall some facts about it and confirm that its deformation is a strictly dissipative process. In the direction of relating force  and deformation  in this element, it is obvious that the deformations of an elastic spring , a dashpot  and a Kelvin-Voigt element  are additive   with  representing an internal force passing through each of these elements. Thus,  where dot over the symbol represents the derivative with respect to time  Eliminating   from these 4 equations complemented with expressions for   and , yields the constitutive relation describing the classic Burgers material

(1)

The four parameters of the model should be determined by a system identification procedure from either stress relaxation or creep experiments usually starting from a virginal state      with a required additional initial conditions either  or

Fig. 1. The Lord Kelvin type of the classical Burgers element.

Namely, in the stress relaxation test  with a prescribed deformation  and   being the Heaviside unit step function i.e. the material has to reach the constant deformation instantaneously and maintain it. Therefore its initial velocity should be infinite as well as its initial acceleration. After reaching  an infinite deceleration should be applied to maintain  Note that   being the Dirac delta, as well as  shown in Fig. 2, perfectly cover this pattern.

Fig. 2. The Heaviside unit step function and its derivatives, see [2].

Therefore in the classical stress relaxation experiments the derivatives in (1) should be taken in sense of distributions, i.e. continuous functions complemented with discontinuous but infinitely differentiable functions. The corresponding measure differential equation reads

(2)

The same pattern of change can be recognized in the creep test with  with prescribed load  and its derivatives also taken in sense of distributions leading to the distributional model of creep, say

(3)

In order to find creep compliance, say  the standard procedure based on the Laplace transform can be applied to (3) yielding

(4)

Note that  is positive on  its derivative exists and is completely monotonic function so   is a Bernstein type function.

In order to find the relaxation modulus corresponding to (1) we apply the Laplace transform to (2), that is

and get

(5)

Note that  so denominator in (5) has two different negative real roots say    and    Rewriting (5) as

and by use of tables given at the end of [3]  we get the relaxation modulus in the form

(6)

It can be shown that (6) is a non-negative and completely monotonic function. However instead of showing so, we recall Theorem 6 of the Hanyga paper [4], stating that if the corresponding creep compliance is a Bernstein function and  (what was already shown) then the relaxation modulus is a locally integrable and completely monotonic function. Thus the classical Burgers viscoelastic model is physically acceptable, i.e. its deformation causes energy dissipation.

2. The forced vibration problem

Consider a rigid block of mass    that can move on the smooth and straight horizontal line under the action of a known disturbing force  The motion of the block is limited since the Burgers viscoelastic body undergoing uniaxial deformation shown in Fig. 1 is attached to it. The other end of the body is anchored to a rigid wall. Therefore (1) holds with   representing both position of the block and the deformation of the body and  is as above. Initially the block is in the state of rest while the viscoelastic body is in its virginal state. One may cast this motion problem into language of equations by use of the Newton axiom and the Newton-Laplace Determination Principle . It reads

(7)

(8)

The equation (7) should be complemented with the constitutive equation describing the classical Burgers material (1). Note that in the Newtonian mechanics it is implicitly assumed that the right hand side of (7) should be a function of time, position and velocity, i.e.  see [5]. However (1) comprises force  and its derivatives  and  as well as velocity  and acceleration  so further preparations are needed. Before, let us introduce the following dimensionless quantities

into (7) and (1). Then, after omitting the bar, one gets more compact form of the problem

(9)

(10)

where prime denotes the derivative with respect to dimensionless time. With this preparation done yet it is not obvious that the problem (9), (10) fits into the classical Newtonian mechanics framework.

Remark: In order to relate constitutive equation (10) with a dynamics equation comprising    used in eq. (6) of [6], Parietti et al. solved (10) with respect to , and integrate it to eliminate the acceleration term yielding in our notation

what is quite complicated expression for the force but it fits into the framework of Newtonian mechanics, see eq. (5) of [6].

In what follows a different procedure will be used to relate (9) and (10) so the framework of Newtonian mechanics for the vibration problem at hand will be easily recognized. Namely, the Laplace transform of (10) for given initial conditions leads to

(11)

Noting that

and introducing negative distinct real roots of  say   and  the straightforward inversion of (11) yields

(12)

with

(13)

Now with    it is obvious that the forced oscillation problem with the Burgers internal friction element fits into Newtonian mechanics framework, since

(14)

to be solved with

3. An example

Consider the above problem with the disturbing force given in the form of linear combination of 5 wavelets of the Ricker type, i.e.

(15)

for ,  and     , shown below

Fig. 3. The Ricker type of disturbing force.

We assume the following values of the Burgers parameters:  N/m corresponding to helical coil springs Ns/m and . Thus       and

The solution of (14) obtained by use of the Bulirsch-Stoer algorithm is shown in the following figure.

Fig. 4. The solution of (14), (15) for N/m and Ns/m.

Note that for  the corresponding velocity satisfies  It can be shown that increasing  the velocity of the block decreases and more energy will be dissipated as expected.

4. Closure

In this paper the forced oscillations with the classical Burgers type of dissipation were examined. It was shown that the problem fits into the framework of Newtonian mechanics. The solution of the problem was obtained by use of the Laplace transform and the convolution theorem. The example with the disturbing force of the Ricker type was given at the end.

Acknowledgments. This research was supported by the Serbian Ministry of Science.

References

- Mainardi F., Fractional Calculus and Waves in Linear Viscoelasticity, An Introduction to Mathematical Models, Imperial College Press, 2010.

- Martinenko V. S., Operational calculs (in Russian), Kiev, 1973.

- Doetch G., Guide to the applications of the Laplace transforms, D Van Nostrand Comp., London, 1961.

- Hanyga A., Physically acceptable viscoelastic models, in Trends in applications of mathematics and mechanics, edited by Wang Y. and Hutter K., Shaker Verlag, Aachen 2005.

- Markeev A.P., Theoretical Mechanics, Moscow, (in Russian) 1999.

- Parietti, F., Baud-Bovy, G., Gatti, E., Riener, R., Guzzella, L., and Vallery H. Series viscoelastic actuators can match human force perception, IEEE/ASME Transactions on mechatronics, Vol. 16, 853-860, 2011.